# S. aureus Electroporation Protocol – Adapted from Plan by Ian R. Monk

### Dan Floyd

- Materials (Have all materials ready before starting preparation of electrocompetent cells_

  - LB Media, no abx

  - LB plates, no abx

  - LB media, 10% glycerol and 500 mM sucrose

    - For 100 ml:

    - 1 g of NaCl

    - 1 g of Tryptone

    - 0.5 g of Yeast Extract

    - 17.5 g of sucrose (for 500 mM sucrose in 100 ml)

    - 13.3 ml of 75% glycerol (for 10% glycerol)

    - 86.7 ml of ddiH2O

  - Sterile ddiH2O (200 ml)

    - Ice cold

  - 10% glycerol in LB (45 ml)

    - Ice cold

    - 6 ml 75% glycerol

    - 39 ml sterile ddiH2O

  - LB plates, 25 ug/ml chloramphenicol, 100 ug/ml XGAL (3 plates per electroporation)

    - 500 ml LB agar

    - 1 ml XGAL, warmed to same temperature as the agar

    - 250 ul 50 mg/ml chloramphenicol stock

  - LB media, 500 mM sucrose (50 ml)

    - 50 ml LB media

    - 8.57 g sucrose

## Preparation of Electrocompetent Cells

- S. aureus must be struck out to single colonies two days prior to the electroporation in order to have overnight cultures ready the day of electroporation

- Prepare overnight cultures of electrocompetent S. aureus cells in 10 ml LB media at 37 °C in the shaking incubator

- These overnight cultures will be diluted down to an OD600 of 0.5 in warmed media, then put back into shaking incubator for another 40 minutes

  - The dilution will be done using the C1V1=C2V2 equation with the concentrations being the OD600s; V2 will be 10 ml and C2 will be 0.5; subtract V1 amount from the 10 ml of LB so total volume equals 10 ml

  - At this step, starting cooling down the big centrifuge

  - If OD600 of any of them is greater than 0.6, dilute it so that it will be less than 0.6

| IF (OD600 Values) | THEN (Dilution) |
| --- | --- |
| 0.6-0.9 | 1:2 |
| 0.9-1.5 | 1:4 |
| 1.6- | 1:10 |

- To harvest cells, spin down at 7000 x g for 5 minutes at 4°C in 50 ml conicles and discard supernatant

- An equal volume of ice-cold sterile water will be added, and the cells spun down again at the same settings (this is done twice)

- The cells will then be centrifuged and resuspended in in 1/5 volume (2 ml) of sterile ice-cold 10% glycerol LB media (this is done twice)

- Finally, the cells will be resuspended in 1/200 (50 ul) sterile ice-cold 10% glycerol LB media and can be stored at -70 °C

- Streak out one aliquot on two plates; add 80 ul LB 500 mM sucrose to 50 ul aliquot to make sure cells can grow

  - 110 ul on LB+CX

  - 20 ul on LB

## Electroporation

- Thaw cells and spin for 3 minutes at 5000 x g, resuspending in 80 ul 10% glycerol-500 mM sucrose LB media

- Plasmid DNA is then added to the cells (1 ug), and then the volume is transferred to a 1 mm electroporation cuvette

  - Allow cells to incubate with the DNA for 30 minutes at room temperature

- After the pulse (at 21 kV/cm, 100 ohms, and 25 µF), 1 ml of LB with 500 mM sucrose is added to the cuvettes and pipetted up and down to mix before being transferred to a tube and incubated at an angle (45 degrees) for 1 hour at 30 °C

- Two 100 ul aliquots are to be pipetted on LB chlor/xgal plates (25 ug/ml chlor, 100 ug/ml XGAL); one plate incubated overnight at 37 °C and the other at 30 C for 48 h

- The remaining cells are to be spun down at 7000 x g for 5 minutes, supernatant removed, and resuspended in 100 ul of the LB 500 mM sucrose before the entire volume is pipetted onto another plate for overnight incubation at 37 C

## Fast Integration

- For plates that have blue colonies (direct integrants), often on a lawn of small white colonies, scrape up six colonies and plate them to single colonies onto a new LB+CX plate, one colony per plate

- Additionally, from the integration plate from the electroporation, inoculate four colonies in 10 ml of LB and incubate at 30 C overnight to stimulate plasmid excision

## Slow Integration

- For strains that do not permit direct integration, inoculate a 200 ul volume of PBS with a colony and then dilute ten-fold to 10^-4.

- Spread 50 ul of the homogenized colony on half a LB+CX plate, and 10 ul spots of the diluted colony on the other half, letting the spots run down the plate. Incubate this plate at 37 C overnight.

- Blue colonies from this plate are to be streaked to single colonies on a new LB+CX plate and incubated at 37 C overnight, along with some blue colonies from this plate to be inoculated in 10 ml LB at 30 C overnight

## Plasmid Excision

- These 30 C overnight cultures are to be diluted down to … and 100 ul plated onto a LB+X plate, incubated at 37 C overnight

- White colonies with successful allelic exchange (confirmed by colony PCR) is streaked to single colony and incubated overnight at 37 C

## Colony PCR

- A very small amount of a white colony is aliquoted into 50 ul of PCR master mix with 200 nM of each primer in the master mix

- 35 PCR cycles are to be run, with the denaturation duration changed from 30 seconds to 3 minutes