Agarose Gel Protocol – written by Joe Paquette

(Note: all ddiH2O is type I)

- Mix 25x TAE buffer with ddiH2O to obtain 1x TAE solution.

- To make 1 L of 1x TAE, add 40 mL 25x TAE into 1 L graduated cylinder and fill to 1 L with ddiH2O.

- Add 1 g agarose powder to 100 mL 1x TAE buffer in 250 mL.

- Add stirbar to container.

- Heat to dissolve the agarose while stirring (don’t let it overboil, should look like clear liquid, no solids).

- Once the sugar has dissolved, make a 1x concentration of SYBR safe in the 1% agarose solution.

- To make, add 10µL of 10,000x SYBR safe dye to the 1% agarose solution

- Let the 1% agarose solution cool to approximately 50-55°C.

- Apply autoclave tape to the edges of the gel cast (ensure the tape is tightly bound).

- Pour 1% agarose - 1x SYBR safe solution into the cast and insert a comb to mold wells in the gel.

- Let sit until the 1% agarose – 1x SYBR safe solution has cooled and solidified into a gel.

- Carefully remove the comb.

- Pour the 100 ml of 1x TAE buffer solution into the gel tank (add just enough to slightly submerge the gel itself).

- Obtain 5 µL of each PCR sample.

- Make 1x loading dye in 6 µL of solution.

- To make 6 µL of 1x loading dye, combine 5 µL PCR sample with 1 µL of 6x loading dye

- Load 10 µL of 2x log ladder into the first well.

- Load 5 µL of the PCR-dye mixture into the wells in sequential order.

- Insert the electrodes and run the gel at about 113 volts.

Materials needed:

- 1.0g of agarose powder

- SYBR safe dye

- 1 packet of 25x TAE buffer mix