Making Measurements Using ImageJ for Zone of Inhibition Assays

- Open ImageJ

- File>Open>20201125_measure.tif

- Select  “Straight”

- Using “Straight” line tool, measure 5 mm on the ruler (between 2 and 2.5), be as precise as possible. Make sure the dots are hitting the lines

- File>Open>[your image file] Drag the file into imageJ. A dialog box might appear, if it does, uncheck ‘Disable Global Calibration’>OK

- Click on ImageJ, MAKE SURE TO CLICK BACK ON THE RULER PICTURE

- Analyze>Set Scale...

- Under ’Known Distance’ type 5.0, under ‘Unit’ type mm, select the ‘Global’ checkbox.

- Using Straight Line tool, measure the diameter of the zone of inhibition (draw a line from the edge of inhibition, through the disc, to another edge)

- Click Analyze>Measure

- While Measure dialogue is open you can draw two more diameters, clicking Measure after each to record the length

- Copy lengths into an Excel spreadsheet

- Repeat with any other images you may have