## Annealing Oligos to Make dsDNA

- Combine in a PCR tube:

| Forward Primer (100 uM in 0.1xEB) | 15 uL |
| --- | --- |
| Reverse Primer (100 uM in 0.1xEB) | 15 uL |
| 1M NaCL | 1.5 uL |

- Anneal oligos in a thermocycler:

95°C for 5 min

Cool to 4°C at a rate of 0.1°C/sec

*Can be stored here at -20°C

- Proceed with digest. If doing a blunt end ligation, will need to use T4 polynucleotide kinase.