## PCR Protocol by John Church (New as of 11/14/18)

- Acquire and label 4 PCR tubes with initials and designate as Tubes 1-4

  - The tubes comes in strips of 8 and they can be split into 4 tube pieces so that the first 3 tubes are used and the fourth is unused

- Get a container of ice to keep the components on

- Acquire the following components and put them on ice, labeling tubes if necessary:

  - ddi H2O in 1.5 mL microfuge tube

  - uL KOD buffer

  - dNTPs

  - oligo F

  - oligo R

  - template

  - Note: KOD enzyme should be kept in the freezer until it is used as it is expensive and should be added last

- Centrifuge the microfuge tubes to get any solution out of the microfuge tube cover

- If any of the solutions are frozen, be sure to vortex the microfuge tube in order to dissolve it (tubes with frozen components may not be homogenized)

  - DO NOT vortex the KOD enzyme itself or any solution with KOD enzyme because vortexing will expose it to oxygen and degrade it

- Use PCR_worksheet.xlsx to make establish the specifics of what will be added

  - The file is located in the Protocols folder

  - For this protocol, a “Total reaction volume” of 100 uL and 4 “Total number of reactions” were used – the following volumes are based on these specifications

- Add 0.75 uL of each experiment specific primer (forward and reverse) to PCR Tubes 1 and 2 (oligos forward and reverse)

  - The amount added should be calculated by taking the total volume for 1 reaction (in worksheet) and subtracting the volumes for 1 reaction that have not yet been added to the master-mix

- Add 0.75 uL of each control primer (oligos forward and reverse) to PCR Tubes 3 and 4

- Add 0.5 uL ddi H2O to PCR Tube 4 so that all 4 PCR Tubes have an even amount of solution

  - Template volume for 1 reaction

- Prepare a master-mix in a 1.5 mL microfuge tube by adding the following according to the worksheet and using micropipettes:

  - Add 27.5 uL ddi H2O

  - Add 27.5 uL dNTPs

  - Add 68.75 uL KOD buffer

  - Add 2.75 uL KOD enzyme

- Mix the master-mix solution by pipetting up and down

  - Do not vortex to mix

- Add 23 uL of master-mix to PCR Tube 4

- Add 2.25 uL template to Master Mix

  - Factor template volume minus 1 template reaction volume.

- Add 23.5 uL master mix to each PCR Tube 1-3 and pipette up and down to mix (conserves tips)

- Close PCR Tubes 1-4 until the caps are tight (push until the caps do not squeak when you push on them)

- Place the PCR Tubes in the thermocycler on STN 1 – the following settings should be in place:

  - Heat at 94 degrees for 2 minutes,

  - 94 degrees C for 20 seconds

  - 50 degrees C for 30 seconds

  - 68 degrees C for 1 minute and 20 second [modified because product is 1334 bp which is over 1 kbp] (KOD polymerase functions properly at 68 degrees C; TAC polymerase is different temp)

  - Go back to step 2

  - Repeat 32x

  - 68 degrees C for 5 minutes

  - 12 degrees C for infinity

## PCR Protocol by John Church (Old)

- Acquire and label 3 PCR tubes with initials and designate as Tubes 1-3

  - The tubes comes in strips of 8 and they can be split into 4 tube pieces so that the first 3 tubes are used and the fourth is unused

- Get a container of ice to keep the components on

- Acquire the following components and put them on ice, labeling tubes if necessary:

  - ddi H2O in 1.5 mL microfuge tube

  - uL KOD buffer

  - dNTPs

  - oligo F

  - oligo R

  - template

  - Note: KOD enzyme should be kept in the freezer until it is used as it is expensive and should be added last

- Centrifuge the microfuge tubes to get any solution out of the microfuge tube cover

- If any of the solutions are frozen, be sure to vortex the microfuge tube in order to dissolve it (tubes with frozen components may not be homogenized)

  - DO NOT vortex the KOD enzyme itself or any solution with KOD enzyme because vortexing will expose it to oxygen and degrade it

- Use PCR_worksheet.xlsx to make establish the specifics of what will be added

  - The file is located in the Protocols folder

  - For this protocol, a “Total reaction volume” of 50 uL and 3 “Total number of reactions” were used – the following volumes are based on these specifications

- Prepare a master-mix in a 1.5 mL microfuge tube by adding the following according to the worksheet and using micropipettes:

  - Add 44 uL ddi H2O

  - Add 44 uL dNTPs

  - Add 110 uL KOD buffer

- Mix the master-mix solution by pipetting up and down

  - Do not vortex to mix

- Add 46 uL of master-mix to each of PCR Tubes 1-3

  - The amount added should be calculated by taking the total volume for 1 reaction (in worksheet) and subtracting the volumes for 1 reaction that have not yet been added to the master-mix

- Add 1 uL template to PCR Tube 3

- Add 1.5 uL of each experiment specific primer (forward and reverse) to PCR Tube 1 (oligos forward and reverse)

  - Note: there are 2 primers, one forward and one reverse, that will be added for a total of 3 uL of primers

- Add 1.5 uL of each control primer (oligos forward and reverse) to PCR Tubes 2 and 3

- Add 1 uL ddi H2O to PCR Tube 3 so that all 3 PCR Tubes have an even amount of solution

- Add 1 uL KOD enzyme to PCR Tubes 1-3 and micropipette up and down to mix

- Micropipette PCR Tubes 1-3 up and down to mix

- Close PCR Tubes 1-3 until the caps are tight (push until the caps do not squeak when you push on them)

- Place the PCR Tubes in the thermocycler on STD 1 – the following settings should be in place:

  - Heat at 94 degrees for 2 minutes,

  - 94 degrees C for 20 seconds

  - 50 degrees C for 30 seconds

  - 68 degrees C for 1 minute (KOD polymerase functions properly at 68 degrees C; TAC polymerase is different temp)

  - Go back to step 2

  - Repeat 32x

  - 68 degrees C for 5 minutes

  - 12 degrees C for infinity