## RNAsnap Protocol

Adapted from: Stead et al. (2012) Nucleic Acids Research

Note: All reagents should be RNAse free; use good RNA-technique by cleaning work area, gloves, and instruments with RNAse-away.

Wear safety goggles when handling formamide and dispose of as hazardous waste.

RNAsnap

- Grow strains to mid-log

- Centrifuge 1 to 10 mL of bacterial culture at 16,000 x g for 30 seconds (or max speed for 3 mins). Discard supernatant.

- Resuspend cell pellet in 100 µL of fresh RES (see recipe below) by vortexing vigorously.

- Incubate sample at 95°C for 7 minutes.

- Pellet cell debris by centrifuging at 16,000 x g for 5 mins at room temperature. Carefully move the supernatant to a fresh tube without disturbing the gelatinous pellet.

Sodium acetate/ethanol precipitation

- Dilute RNA sample to 300 µL with water.

- Add 1/10 volume of 3 M sodium acetate, pH 5.2 (30 µL).

- Add 2 µL GlycoBlue Coprecipitant (ThermoFisher)

- Add three volumes of 100% ethanol (900 µL). Mix by vortexing then incubate for at least 60 mins at -80°C.

- Centrifuge at 16,000 x g (or max speed) for 30 mins at 4°C. Carefully remove supernatant.

- Wash cell pellet with 250 µL of freshly-diluted 75% ethanol.

- Centrifuge at 16,000 x g (or max speed) for 5 mins at 4°C. Carefully remove supernatant.

- Briefly centrifuge again to allow removal of all remaining ethanol, then air dry the pellet.

- Resuspend the pellet in 80 µL water. Centrifuge at 16,000 x g for 1 minute to pellet any remaining water insoluble proteins. Transfer the RNA-containing supernatant to a fresh tube.

- Check concentration by Nanodrop (RNA setting).

DNAse Treatment (Promega)

- Add 10 µL RNase-free DNase buffer and 10 µL RNase-free DNase (Promega, RQ1)

- Incubate at 37°C for 1 hour.

- Repeat sodium acetate/ethanol precipitation as above (steps 6-14).

### Buffers and Reagents:

For all reagents (except 2-mercaptoethanol), use specific stocks for RNA work.

RNA Extraction Solution (RES)

| Component | Stock Solution | Final Concentration | Volume (µL) |
| --- | --- | --- | --- |
| Water |  |  | 5.3 |
| Formamide | 100% | 95% | 1900.0 |
| EDTA | 0.5 M | 18 mM | 72.0 |
| SDS | 20% | 0.025% | 2.5 |
| BME | 99% | 1% | 20.2 |
|  |  | Final volume: | 2000.0 |