## Purification of OMV/T

3/4/20, KMR

Prepare 1 L baffled flask with up to 200 mL sBHIc

Inoculate flasks with bacteria and grow until either exponential (OD600 = 0.6) or early stationary (OD600 = 1.4).

Spin bacteria at 4°C at 4,000 rpm for 30 minutes in 50 mL conicals.

Filter-sterilize supernatant using 0.2 micron filter.

Ultracentrifuge supernatant in a 50.2 Ti rotor at 30,000 rpm (108471.9 x g) for one hour.

Resuspend pelleted OMV in 1x SLB.