# Arbitrary PCR

1) Purify gDNA from transposon insertions of interest.

2) Prepare a table indicating which samples are being tested. Always include LVS genomic DNA (no transposon insertion) and no template control.

Example:

| Sample number | DNA |
| --- | --- |
| 1 | pSD26 Tn insertion 1 (Tn3) |
| 2 | pSD26 Tn insertion 2 (Tn6) |
| 3 | LVS gDNA |
| 4 | -DNA |

3) Set up reaction 1, using 500 ng of gDNA in a total reaction volume of 25 uL. See worksheet for details.

Example:

| Reaction 1 | # of reactions here: |  |  | 4 |
| --- | --- | --- | --- | --- |
|  | # of reactions allowing for error: |  |  | 5.3 |
|  |  |  | Volume of each component (uL) |  |
| Component | Stock Concentration | Final Concentration | Per reaction | Master Mix |
| ddiH2O | - | - | 14.245 | 75.5 |
| OneTaq Rxn Buffer | 5X | 1X | 5 | 26.5 |
| dNTPs | 10 mM | 0.2 mM | 0.5 | 2.7 |
| KROL87 Arb1 (or KROL89 Arb6?) | 100 µM | 4.0 µM | 1 | 5.3 |
| Internal Specific Primer: KROL90 Tn_Mar1 | 10 µM | 0.4 µM | 1 | 5.3 |
| Template | 500 ng | 160 ng/µL | 3.13 | - |
| OneTaq Polymerase | 5 units/uL | 6.25 units | 0.125 | 0.7 |
| Total volume |  |  | 25 | 132.5 |
| Thermocycler program: | Step | Temp | Time |  |
|  | 1 | 95°C | 5' |  |
|  | 2 | 94°C | 30" |  |
|  | 3 | 30°C | 30" |  |
|  | 4 | 72°C | 60" |  |
|  | 5 | Go to step 2, repeat 5x |  |  |
|  | 6 | 94°C | 30" |  |
|  | 7 | 65°C | 30" |  |
|  | 8 | 72°C | 2' |  |
|  | 9 | Go to step 6, repeat 29x |  |  |
|  | 10 | 72°C | 5' |  |
|  | 11 | 12°C | hold |  |

4) Dilute reaction 1 PCR samples 1:10 (2 uL into 18 uL). Keep this dilution for future PCRs!

5) Set up reaction 2, reference worksheet for details.

Example:

| Reaction 2 | # of reactions here: |  |  | 4 |
| --- | --- | --- | --- | --- |
|  | # of reactions allowing for error: |  |  | 5.3 |
|  |  |  | Volume of each component (uL) |  |
| Component | Stock Concentration | Final Concentration | Per reaction | Master Mix |
| ddiH2O | _ | _ | 16.625 | 88.1 |
| OneTaq Rxn Buffer | 5X | 1X | 5 | 26.5 |
| dNTPs | 10 mM | 0.2 mM | 0.5 | 2.7 |
| KROL88 Arb2 | 100 µM | 4.0 µM | 1 | 5.3 |
| External Specific Primer: KROL92 Tn_Mar3 | 10 µM | 0.4 µM | 1 | 5.3 |
| PCR 1 Amplicon, diluted 1:10 | - | - | 0.75 | - |
| OneTaq Polymerase | 5 units/uL | 6.25 units | 0.125 | 0.7 |
| Total volume |  |  | 25 | 132.5 |
| Thermocycler program: | Step | Temp | Time |  |
|  | 1 | 94°C | 30" |  |
|  | 2 | 65°C | 30" |  |
|  | 3 | 72°C | 2’ |  |
|  | 4 | Go to step 1, repeat 29x |  |  |
|  | 5 | 72°C | 5' |  |
|  | 6 | 12°C | hold |  |

6) Run PCR on gel and confirm that there are products. If so, proceed to next step. If not, optimize primers and annealing temperatures further.

7) Re-do PCR 2 using a total reaction volume of 50 uL.

8) PCR-purify samples from larger reaction 2 and elute in 0.1x EB.

9) Submit to the GSC for sequencing using KROL92 Tn_Mar3 (or transposon-specfic primer used in reaction 2).

10) Identify where each transposon insertion is located and design strain-specific primers to confirm specific insertions.