### Dialysis Cassette Protocol

For use with Slide-A-Lyzer Dialysis Kit; 3,500 MWCO, 0.5-3 mL

In advance: Have 1 L of dialysis solution prepared in a 1 L beaker with a stir bar. Ensure there is a stir plate at 4C to use overnight.

Hydrate Membrane:

- Remove cassette from its pouch and slip into groove of buoy.

- Immerse cassette in dialysis buffer for 1-2 minutes (2K cassettes, at least 2 mins)

- Remove cassette from buffer and remove excess liquid by tapping the edge of the cassette gently on paper towels. DO NOT BLOT THE MEMBRANE.

Add Sample:

- Attach the hypodermic needle to the syringe by screwing on luer-lock.

- Fill the syringe with a small volume of air, then immerse the needle in the sample and slowly draw back the syringe to fill with sample.

- Remove cassette from the buoy. Penetrate gasket through one of the syringe ports at a corner of the cassette with the needle and inject sample. Mark/record the injected port. DO NOT ALLOW THE NEEDLE TO CONTACT THE MEMBRANE.

- With the needle still in the cassette cavity, draw up the piston to remove air and to compress the membrane so the sample contacts the greatest surface area. A small amount of air left is okay.

- Remove the syringe from the cassette while retaining air in the syringe.

- Slip the cassette into the groove of a buoy and float this assembly in the dialysis solution of choice.

- Put on a stir plate at 4C overnight.

Remove Sample:

- Fill the syringe with a volume of air at least equal to the sample size. For low-volume samples, fill with two-times the sample volume.

- Penetrate the gasket with the needle through a top, unused syringe guide port. Discharge air into cassette cavity to separate membranes, which prevents needle penetration of the membrane.

- Turn the cassette so the sample collects near the needle. Withdraw the sample with the syringe.