Real-Time Primer Efficiency Test

Note:  Efficiencies should be run on same “type” of DNA as you plan to do the experiment on; i.e. ChIP= sonicated DNA, RNA expression = cDNA or RNA w/ 2-step kit

- Determine concentration of DNA

- Make a dilution that’s final concentration is 1.5ng/ul

- Make 1:10 serial dilutions so you have 4 different DNA concentrations (1.5ng/ul, 0.15ng/ul, 0.015ng/ul, 0.0015ng/ul); you will be using 5ul of these DNA dilutions for each rxn so you need 3ul ddH2O per reaction instead of 7ul

- Put 17.5ul of each DNA sample into separate tubes (enough for 3.5rxns)

5)  set up primer master mixes in separate eppendorfs (make one for each primer pair)

Since you have 4 DNA samples you have 14rxns/primer pair (add 3.5 to this for 		volume error) = 17.5

example:

10ul of SYBR green               x 17.5   =  175

1ul of 5uM primer set		x 17.5   =  17.5

4ul ddH2O                             x 17.5    =  70

TOTAL =  262.5

- Since 3.5 rxns x 20ul rxn volume = 70; and tubes already have 17.5ul of DNA in them you need to add 52.5ul (70-17.5) of primer master mix to each DNA tube

- Pipette 20ul of each rxn into 3 separate wells on the 96 well plate

- Spin plate down

- Place in real-time machine and run rxn

Calculations

ChIP enrichment

Primer Efficiency ^ (Ct_diff.)

Ct_diff.  = (control_ChIP_Ct –test_ChIP_Ct) - (control_Input_Ct – test_Input_Ct)

Primer Efficiency =  (10^(-1/slope))

Where the slope is the slope of the standard curve plot

GSC computer login:

K_Ramsey Lab

Password:

Roche480