# RNA Stability Assay

Adapted from protocol sent by Dr. Scarlet Shell, WPI

- Prior to the day of the experiment, streak strains of interest to single colony and patch onto CHAH+selective antibiotic, according to the appropriate growth time of the strains

- Resuspend patches of strains of interest in 400 uL of MHB and check OD in a 1:20 dilution

- Start cultures at an appropriate OD (0.08 for WT background or 0.10 for strains with a growth defect) in 40 mL of culture + appropriate antibiotic and grow to mid-log (OD 0.3-0.4), typically ~4-4.6 hours

- While cultures are growing, label and prepare 15 mL tubes and prepare 0.5 mg/mL rifampin in DMSO

- Once OD ~0.3, transfer 7mL of each replicate into appropriately labeled 15 mL tube, one for each time point and allow to shake for an additional 30 minutes

- After 30 minutes, flash freeze 0’ time point in liquid nitrogen, then add rifampin to the tubes to a final concentration of 0.5 ug/mL (7uL of 0.5 mg/mL rif) to each subsequent tube of a given replicate, flashing freezing each 15 mL tube after either 1, 2, 4, or 8 minutes.

- Repeat for each biological replicate, can start second time course during 4 minute wait, cultures can be stored in the -80 for up to two months

- When prepared, thaw cultures and proceed with RNA extraction and purification, cDNA synthesis, and qRT-PCR protocols

- For analysis, preform linear regression on a plot of -CT over time. Half-life = −1/slope. If there is a delay in decay, the first or second time point can be removed from the linear regression. If there is a plateau after an exponential decay, timepoints after plateau can be removed.

| Sample Experimental Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Strain | Strain 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Strain 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Replicate | 1 |  |  |  |  | 2 |  |  |  |  | 3 |  |  |  |  | 1 |  |  |  |  | 2 |  |  |  |  | 3 |  |  |  |  |
| rif (min) | 0 | 1 | 2 | 4 | 8 | 0 | 1 | 2 | 4 | 8 | 0 | 1 | 2 | 4 | 8 | 0 | 1 | 2 | 4 | 8 | 0 | 1 | 2 | 4 | 8 | 0 | 1 | 2 | 4 | 8 |