# Running RNA Gel

Written by: Sierra Schmidt

- Make 10 uL aliquots with normalized concentrations, using water to dilute. Then add 2 uL purple loading dye for a final concentration of 1-1.5x purple loading dye and run on gel.

  - Note: RNA concentration should end with each sample being pipet-able. Typically ran final concentration of ~300 ng/uL

- Rinse comb and gel rig with DI water, then spray with ethanol

- Make fresh 1% agarose gel by adding 1.2 g to 120 mL fresh 1xTAE (or 0.6 g in 60mL 1xTAE for small gel rig) and stir with heat until completely dissolved, then place in 56°C water bath until cool enough to touch.

- Set up gel rig to cast gel, with ladder.

- Add 12 uL of Sbyr Safe dye to rig, pour gel, use ladder to mix, then replace ladder and allow to set.

- Add new 1xTAE, turn gel, and remove ladder.

- Load 12 uL of each sample.

- Ran until separated at 113V, at least ~15 minutes.

- Look for distinctive bands (23s, 16s, tRNA+5s), rather than smears.