## Generate cDNA (half protocol)*

*REACTION SIZE CUT IN HALF from Lory lab microarray protocol

Combine the first components for primer annealing:

| Component | Volume or Amount | Final Concentration |
| --- | --- | --- |
| RNA | 3 ug | 267 - 333 ng/ ul |
| (NS)5 Primer (250 ng/ul) | 1.5 ul | 25 ng/ul |
| RNase-free water | up to 15 ul |  |

To normalize all the cDNA samples to the same DNA mass, dilute with RNase-free water in PCR strip tubes.

Example sample table with volumes of cDNA and water

|  |  |  |  |  | cDNA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | Sample ID |  | Date isolated | RNA Conc (ng/ul) | RNA | H2O | Total RNA |
| 1 | 1A | LVS | 8/4/20 | 424.9 | 7.06 | 6.44 | 3000.0 |
| 2 | 1B |  |  | 419.7 | 7.15 | 6.35 | 3000.0 |
| 3 | 1C |  |  | 449.9 | 6.67 | 6.83 | 3000.0 |
| 4 | 2A | LVS FTL_0146(F315L) |  | 507.3 | 5.91 | 7.59 | 3000.0 |
| 5 | 2B |  |  | 551.9 | 5.44 | 8.06 | 3000.0 |
| 6 | 2C |  |  | 522.7 | 5.74 | 7.76 | 3000.0 |
| 7 | 3A | LVS ∆pmrA |  | 422.7 | 7.10 | 6.40 | 3000.0 |
| 8 | 3B |  |  | 292.9 | 10.24 | 3.26 | 3000.0 |
| 9 | 3C |  |  | 315.6 | 9.51 | 3.99 | 3000.0 |
| 10 | 4A | LVS ∆pmrA FTL_0146(F213L) |  | 472.8 | 6.35 | 7.15 | 3000.0 |
| 11 | 4B |  |  | 511.5 | 5.87 | 7.63 | 3000.0 |
| 12 | 4C |  |  | 513.8 | 5.84 | 7.66 | 3000.0 |
| 13 | 5A | LVS ∆pmrA(sup) |  | 355.6 | 8.44 | 5.06 | 3000.0 |
| 14 | 5B |  |  | 422.2 | 7.11 | 6.39 | 3000.0 |
| 15 | 5C |  |  | 537.3 | 5.58 | 7.92 | 3000.0 |

Note the total volume of cDNA and water is 13.5 uL.

Add the 1.5 uL (NS)5 oligo (stored in small aliquots at -80°C; take your own aliquot to use and move it to your own stock box; minimize freeze/thaw cycles) to the tubes.

Incubate using program JSScDNA1 in the thermocycler:

Step	Temp	Time

1	70°C	10'

2	25°C	10'

3	4°C	hold

While waiting, prepare the cDNA synthesis reaction in master mix format. The number of reactions you have plus 1.5 should be the factor you multiply by.

Example master mix calculations- this master mix would be enough for 21 cDNA reactions:

| Component | Final Concentration | Volume | x 22.5 |
| --- | --- | --- | --- |
| 5X 1st strand buffer | 1x | 6 | 135 |
| RNase-free water |  | 2.88 | 64.8 |
| 100 mM DTT | 10 mM | 3 | 67.5 |
| 10 mM dNTPs | 0.5 mM | 1.5 | 33.75 |
| Superscript III (200 U/ul) | 10.8 U/ul | 1.63 | 36.68 |

*Buffer, DTT, and dNTPs should be from the cDNA reagents box (they are RNase free!)

Aliquot 15 ul of master mix per reaction into each PCR tube from the first reaction (total volume now 30ul)

Incubate using program JSScDNA2

Step	Temp	Time

1	25°C	10'

2	37°C	60'

4	42°C	60'

5	70°C	10'

6	4°C	hold

*potential stopping point- samples can be stored at -80°C if necessary

Remove RNA from sample by degrading with sodium hydroxide:

Add 10 ul of 1N NaOH

Incubate 65°C for 30'

Neutralize with 10 ul of 1N HCl

Final volume is 50 ul

Purify cDNA using Qiagen PCR clean-up column (be very sure there is no ethanol carry-over!)

Elute in 60 ul of 0.1x EB

Check concentration by Nanodrop

Store cDNA at -80°C